CSC 364 – Computer Architecture

Homework #3

- A computer memory system is addressable by a 28 bit address bus. The cache in this system supports direct addressing, with a 4-bit word, and 19-bit line, and a 5-bit Tag. Answer the following questions:

  - What is the size of this memory system?

  - What is the size of cache in this system?

  - How many words are in each cache line?

  - What is the maximum number of different tags that can be loaded in this cache

- Consider a machine with a byte addressable main memory of 216 bytes and a block size of 8 bytes. Assume that a direct cache consisting of 32 lines is used with this machine.

  - How is the 16-bit memory address divided into Tag, Line number, and byte number

  - Into what lines would bytes with each of the following addresses be stored:

    - 0001 0001 0001 1011

    - 1100 0011 0011 0100

    - 1101 0000 0001 1101

    - 1010 1010 1010 1010

  - How many total bytes of memory can be stored in the cache?

- For the hexadecimal main memory addresses 444444, 999999, CCCCCC, show the following information:

  - Tag, Line, and Word values for a direct mapped cache using the format of (8-bit tag, 14-bit line and 2-bit word)

  - For an associative-mapped cache, with a 22-bit tag, what are the tag and word values

- Consider a CPU with the following specifications:

- It can access 256 words of memory, each word being 8 bits wide. The CPU does this by outputting a 8-bit address on its output pins A[7…0] and reading in the 8-bit value from memory on its inputs D[7…0]

- The CPU contains an 8-bit address register (AR), program counter (PC), accumulator (AC), data register (DR), and a 4-bit instruction register (IR).

- The CPU must realize the following instruction set. Note that β is an 8-bit value stored in the location immediately following the instruction

| Instruction | Instruction Code | Operation |
| --- | --- | --- |
| LD | 000XXXXX β | AC ← M[β] |
| STI | 001XXXXX β | β ←AC |
| ADD | 0100XXXX β | AC ← AC + M[β] |
| JUMP | 1000XXXX β | PC ←β |
| SKIP | 1100XXXX | PC ←PC + 1 |
| RST | 1110XXXX | PC ←0, AC ←0 |

Write and Design the following parts for this CPU:

- RTL (Register Transfer language)

- Data path / Register Section